| Data: | Spatial: Contains the raw datasets, as downloaded from each source (without any pre-processing). | Tabular: Contains the crosswalks tables used to harmonize the data in the pre-processing workflow. |  |
| --- | --- | --- | --- |
| Intermediate: | Contains any intermediate data layers created throughout the pre-processing workflow, organized into folders. | Currently, they’re mostly all on an external drive, as I was running low on storage on my Google account. If you need any layers before we set up the new data storage solution, please let me know. |  |
| Output: | Contains the output layers needed for data visualization and/or MST analyses. This is where you can find the following raster files |  |  |
|  | pither_model.tif: resistance layer from Pither et al., cropped and masked to the modelling extent. I believe you’ll need this to work through Wenyuan’s script. pa_model.shp: the filtered set of protected areas across the modelling extent. For testing purposes, if you need to filter it further down to fewer protected areas, let me know and I can point you to the easiest point along the code to do so. dem_model.tif: DEM, cropped and masked to the modelling extent. |  | lulc_road_model.tif: harmonized LULC map, cropped and masked to the modelling extent. This file will be updated soon to finalize the representation of grassland in the Quebec portion of the extent, but it already includes all the classes. Here is a table summarizing class IDs to names. Updates will also be made to represent forestry roads differently from minor roads. cover_type_model.tif: breaks down the class “Forest” into mixed, deciduous and coniferous. age_model.tif: breaks down the class “Forest” into age classes. This is a first pass on the age classes. You can check the harmonization table here (slide 11). Once you get into building the habitat suitability model for each species, let us know which adjustments you think should be made to the age classes, if any. |

Task 4. Construct species-specific MSTs for protected areas

Objective: Develop protected area Minimum Spanning Trees (MSTs) using species-specific movement cost surfaces and dispersal thresholds.

Key steps:

- Define nodes and node representation based on a set of protected area

  - PAs of various IUCN classes, >10 km2 in size and > 50 m apart. Aquatic PAs removed excluded.

- Model landscape movement costs for each species

  - Species selected in consultation with the MELCCFP, and using a ranking system to capture a set of focal species from several taxa that capture the range of dispersal capabilities.

  - Movement costs scored in-part using Pither et al. 2023 and literature review with expert opinion on habitat suitability of land cover types, forest classification, and forest age.

- Derive pairwise link costs for each species

  - Using R and terra:costDist() function to get pairwise link costs for each species, depending on their dispersal capacity as the maximum cutoff distance.

- Build MST for each species

  - Using Prim’s algorithm and the igraph::mst() function to get the minimum spanning tree (or forest) of the connected PAs for each species.

- Apply dispersal kernel thresholding

- Assess sensitivity to dispersal thresholds

- Prepare standardized outputs per species

- Document methods

Key outputs: Resistance surfaces, Tabulated dispersal parameters, MST maps for each focal species (raster and vector maps); sensitivity analyses of dispersal thresholds (summary tables); documented workflows (Word document).

Timing: May-August 2026

KEY STEPS:

Species specific dispersal threshold for the PA pairs – based on what species traits (dispersal distance)

- Try to include other smaller PAs to get a network to get smaller inter-patch distances

- Maybe a set of smaller PAs for the ‘small’ and ‘large’ species

- Instead of dispersal distance, use a negative exponential kernel to capture it as a cutoff (to capture the extreme movements by individuals) – maybe use the range of values instead of the median value (as is reported in the species list)

  - Dispersal flux between nodes i and j separated by a distance dij (i.e., the probability that an individual in node i will disperse to node j) was calculated as a negative exponential kernel, Pij = exp[dij × log(0.5)/D50], where D50 is the species-specific median dispersal distance (From (Albert et al. 2017).

Scoring the movement cost layer – what parameters and range of values are we using to inform the ‘cost layer’

- Data driven approach – generate a habitat suitability, informed using the relevant layers on the google drive folder

  - What raster files are relevant to the habitat suitability

- Include biodiversity observation from https://biodiversite-quebec.ca/en? Or GBIF?

  - Natureserve https://explorer.natureserve.org/pro/Welcome/

  - https://www.biodivcanada.ca/topics/monitoring-and-data

  - https://www.donneesquebec.ca/recherche/fr/dataset

- Suitability ‘expert-opinion’ literature review approach

  - Parameters defined from Carina’s species-selection approach, including habitat associations and dispersal distances

  - Following methods from Albert et al. 2017

“We assigned numerical resistance values to different landscape features and classified them into 5 classes (Table S4). We distinguished between land cover types that were considered to have low resistance to the movement of our focal species (high permeability; e.g. forests), intermediate resistance (e.g. agricultural fields), and high resistance (low permeability; e.g. highways; Table S4). We also accounted for linear elements such as hedges, drainage ditches and exterior forest edge (open areas within a given distance to forest; Fig. M2.1) that can enhance permeability in agricultural fields (Maisonneuve 2001). For landscape features that are not recorded in Table S4 we used a reclassification of the inverse of habitat quality (100-99/60*habitat quality) to give high resistance values to built-up urban areas. Preferred habitat had a resistance value of 1 and subsequent resistance values doubled between classes (i.e. 2, 4, 8, 16, 32; Table S4). “

Task 5. Integrate MST networks with species distributions

Objective: Integrate MSTs with species-specific distribution maps to identify functionally connected and suitable areas for each focal species. Optional to overlay MSTs with additional species current distributions. Or optional to overlay with future distributions for focal species.

Lead: Postdoctoral researcher

Contributors: Firkowski (data preparation and post-processing), Rayfield (integration and interpretation)

Key steps:

5.1.   Align MST outputs with distribution layers

5.2.   Define integration rules (e.g. overlap, minimum suitability thresholds, etc)

5.3.   Identify functionally connected suitable areas

5.4.   Assess sensitivity to integration rules

5.5.   Prepare standardized outputs per species

5.6.   Document methods

Key outputs: Integrated MSTs for focal species (raster and vector maps).

Timing: September-October 2026

Task 6. Assess protection status and vulnerability

Objective: Evaluate the extent to which integrated MSTs are protected and assess changes under alternative protected area scenarios.

Lead: Postdoctoral researcher

Contributors: Firkowski (data preparation and post-processing), Rayfield (integration and interpretation)

Key steps:

6.1.   Overlay integrated MST outputs with protection layers

6.2.   Classify connectivity vulnerability

6.3.   Evaluate baseline protection coverage

6.4.   Assess alternative protection scenarios

6.5.   Produce spatial and summary outputs

Key outputs: Quantitative metrics of protection coverage and vulnerability (summary tables, raster and vector maps); comparative analyses across species and scenarios (summary table).

Timing: November-December 2026

Task 7. Prepare report of results and maps

Objective: Synthesize analytical methods and results into a comprehensive report and supporting spatial products.

Lead: Postdoctoral researcher

Contributors: Project team

Key outputs: Draft English-language report (Word document); final figures (high resolution images); archived data (tables, raster and vector maps) and scripts (code).

Timing: November-December 2026